Milestone 1 — Client Design Review Home Affairs Regional Office (Klerksdorp)

| STUDENT NAME SIBONISO MPEHLA | STUDENT ID 37277022 |
| --- | --- |
| CLIENT ID / PROJECT ID CLI-093 ¦ CMPG325-2026-093 | ASSIGNED NET BLOCK 10.38.0.0/16 |
| TECHNICAL CHALLENGE Port Security (Switchport Access Control) | CHANGE REQUEST CR5 (25% Host Growth Absorption) |

1. Client Requirements

Client & Location: Home Affairs Regional Office (Klerksdorp) [Client ID: CLI-093 | Project ID: CMPG325-2026-093].

Network Segment: Assigned network block 10.38.0.0/16.

Change Request (CR5): Accommodate 25% host growth absorption for Staff Operations by allocating a /23 subnet (10.38.20.0/23).

Technical Challenge (Port Security): Enforce MAC address limits and security violation actions (restrict for Staff, shutdown for Public) on switch access ports.

Inter-VLAN Routing: Implement Router-on-a-Stick (802.1Q encapsulation) to enable secure communication across subnets.

2. Physical Topology

Router (R1-CORE): Cisco 2911 / 4331 router serving as the Inter-VLAN gateway.

Switch (S1-ACCESS): Cisco Catalyst 2960 switch managing Layer 2 access ports and trunking.

End-User Devices:

• PC-IT-Admin: Connected to Switch Port Fa0/2.

• PC-Staff-01: Connected to Switch Port Fa0/3.

• PC-Public-01: Connected to Switch Port Fa0/4.

• Server-Main: Connected to Switch Port Fa0/5.

Physical Cabling: Copper Straight-Through cables connecting end devices to switch ports, and link between S1-ACCESS (Fa0/1) and R1-CORE (G0/0).

3. Logical Topology

VLAN 10 (Management_IT): IT administration segment.

VLAN 20 (Staff_Operations): Operations network expanded under CR5 headroom allocation.

VLAN 30 (Public_Counters): Public access segment with strict port security rules.

VLAN 40 (Restricted_Servers): Dedicated server room segment.

Trunk Connection: 802.1Q trunking on port Fa0/1 carrying traffic for VLANs 10, 20, 30, and 40 to router sub-interfaces (G0/0.10 to G0/0.40).

4. IP Addressing Plan

| Device / VLAN | VLAN ID | Subnet / CIDR | Subnet Mask | Usable Host Range | Default Gateway |
| --- | --- | --- | --- | --- | --- |
| Management_IT | 10 | 10.38.10.0/24 | 255.255.255.0 | 10.38.10.1 - 10.38.10.254 | 10.38.10.1 |
| Staff_Operations | 20 | 10.38.20.0/23 | 255.255.254.0 | 10.38.20.1 - 10.38.21.254 | 10.38.20.1 |
| Public_Counters | 30 | 10.38.30.0/24 | 255.255.255.0 | 10.38.30.1 - 10.38.30.254 | 10.38.30.1 |
| Restricted_Servers | 40 | 10.38.40.0/27 | 255.255.255.224 | 10.38.40.1 - 10.38.40.30 | 10.38.40.1 |

Configured End-Device IP Assignments:

PC-IT-Admin: 10.38.10.10/24 | Default Gateway: 10.38.10.1

PC-Staff-01: 10.38.20.10/23 | Default Gateway: 10.38.20.1

PC-Public-01: 10.38.30.10/24 | Default Gateway: 10.38.30.1

Server-Main: 10.38.40.10/27 | Default Gateway: 10.38.40.1

5. Initial GitHub Repository

Repository Name: CMPG325-Milestone1-CLI093

Repository URL: https://github.com/Siboniso-Mpehla/CMPG325-Milestone1-CLI093

Repository Structure:

• README.md (Project overview, client specifications, topology details)

• /configs/S1-ACCESS.cfg (Switch Cisco IOS startup configuration script)

• /configs/R1-CORE.cfg (Router Inter-VLAN Cisco IOS configuration script)

• /topology/Home_Affairs_Klerksdorp.pkt (Cisco Packet Tracer simulation file)

• /docs/Milestone1_Client_Design_Review.docx (Documentation report)